SOLAR – B   EIS Radiator Thermal Study	MSSL

Preliminary study of radiator positioned mid-way along EIS

- CCD temperature aim:	-80C

- CCD dissipation:			2 x 0.5W

- No other dissipation considered

EIS  CONFIGURATION STUDIED

THERMAL ENVIRONMENT and ASSUMPTIONS

- Orbit: 600 x 600 km, i = 97.8, 6pm ascending node.

- Winter Solstice (provisional hot case)

- Spacecraft Sun-pointing

- Total isolation of radiator from structure assumed

- No heat load from camera electronics

EIS Radiator Preliminary Thermal Model (1)

EIS Radiator Preliminary Thermal Model (2)

###### CONFIGURATIONS ANALYSED

- 190 x 230 mm radiator

- With Earth shield around radiator

- Radiator angled away from spacecraft

- Radiator extensions on +Y and –Z faces

- Additional extensions –Z side of main radiator

- 190 x 230 mm radiator at –Z end of EIS

THERMAL PREDICTIONS

Radiator temperature predictions (max. and min. around orbit):-

Note:	Model has low heat capacity - true orbital variations will reduce.

Config. 1 (baseline)				-80C to -12C 	(average -45C)

Config. 2 (Earth shields)	 		-70C to -10C 	(average -40C)

Config. 3 (angled radiator)		-83C to -7C 	(average -45C)

Config. 4 (radiator extensions)	-70C to -30C 	(average -50C)

Config. 5 (additional extensions)	-70C to -30C 	(average -50C)

Config. 6 (rear radiator)			-65C to -45C 	(average -55C)

Comparison: XRT				-75C to -55C 	(average -65C)

DISCUSSION

- Current radiator position seems inadequate for -80C on CCD

- Earth shields and alternative thermal finishes are not effective

- due to blockage of view factor to Space by rear of EIS and spacecraft

Radiator on rear face of EIS is more effective due to uninterrupted view to Space – does receive more Earthshine & Albedo

- Optimisation (pointing angle, Earth shields) should reduce temperatures to -65C average, in line with XRT analysis.

WAY FORWARD

- Optimise current radiator with extensions / active louvres

- Unlikely to achieve -80C

- Heat pipe link to rear radiator

- Not enough power to activate heat pipe?

- Peltier (Thermo-electric) cooler and heat pipe to rear radiator

- Power, mass & complexity implications

- Redesign instrument with rear-mounted radiator

 	Review -80C CCD temperature requirement